Speed-Reactive Radar Scanner using ESP32

1. Aim

To build a radar-style scanner that sweeps an ultrasonic sensor from side to side using a servo motor. The special part is that the scanner reacts to how close an object is: when something comes near, the servo sweeps faster, and when everything is far away, it sweeps slowly. A live web page shows the distance and the current scanning speed.

2. What is an ESP32? (Quick Note)

An ESP32 is a small computer board, a little like an Arduino, but it can also connect to Wi-Fi. Here it does three jobs at once: it reads the distance sensor, moves the servo motor, and runs a small web page you can open on your phone.

3. How Does the Distance Sensor Work?

The HC-SR04 sensor works like a bat. It sends out a sound that is too high for us to hear. That sound travels, hits an object, and bounces back. The sensor measures how long the sound took to come back. Sound travels at a known speed, so from the time we can work out the distance.

Because the sound goes to the object AND comes back, we divide the answer by 2 to get the real one-way distance.

4. Parts Needed

| Part | How Many |
| --- | --- |
| ESP32 Board | 1 |
| HC-SR04 Ultrasonic Sensor | 1 |
| SG90 Servo Motor | 1 |
| Jumper Wires | 1 set |
| USB Cable | 1 |

5. How to Connect the Wires

| Part / Pin | Connect to ESP32 |
| --- | --- |
| HC-SR04 - TRIG | GPIO 5 |
| HC-SR04 - ECHO | GPIO 18 |
| HC-SR04 - VCC | 5V |
| HC-SR04 - GND | GND |
| Servo - Orange (signal) | GPIO 4 |
| Servo - Red (power) | 5V |
| Servo - Brown (ground) | GND |

Important: The ECHO Wire

The ECHO pin sends back a 5V signal, but ESP32 pins are made for only 3.3V. On real hardware this can damage the pin over time. Two easy fixes:

- Use two resistors (1k and 2k) as a voltage divider on the ECHO wire to bring 5V down to about 3.3V.

- Or use an HC-SR04P sensor, which already works at 3.3V and needs nothing extra.

Important: Servo Power

In this project the servo never stops moving, and it moves very fast when an object is close. That uses a lot of power. If your ESP32 keeps restarting while the servo runs, that is a power problem, not a code problem. Give the servo its own 5V supply, and connect all the grounds together (servo ground, supply ground, and ESP32 ground).

6. Software Needed

- Arduino IDE (the app to write and upload code)

- ESP32 board package added inside Arduino IDE

- ESP32Servo library (installed from Library Manager)

- A phone or laptop on the same Wi-Fi to open the web page

7. How the Project Works (Simple Idea)

- The servo turns the sensor step by step, from 0 degrees to 180 degrees and back again.

- At every step, the sensor measures how far away the nearest object is.

- The code turns that distance into a speed. Object close means the servo waits less between steps, so it sweeps fast.

- Object far away means the servo waits longer between steps, so it sweeps slowly.

- The ESP32 also runs a web page showing the live distance and speed. The page turns red when something is closer than 15 cm.

8. The Main Idea: Distance Becomes Speed

This is the heart of the project. The trick is simple once you see it:

- A SMALL delay between steps means the servo moves FAST.

- A BIG delay between steps means the servo moves SLOWLY.

So the code just works out how big the delay should be, based on the distance:

- Object at 5 cm (very close) gives the smallest delay, about 5 ms, so the sweep is fastest.

- Object at 50 cm (far away) gives the biggest delay, about 40 ms, so the sweep is slowest.

- Anything in between gets a delay in between. Move your hand closer and you can watch it speed up.

9. The Code

#include <ESP32Servo.h>

#include <WiFi.h>

#include <WebServer.h>

// ---------------- WiFi Configuration ----------------

const char* ssid = "NSUT_WIFI";

const char* password = "";

WebServer server(80);

// ---------------- Hardware Pins ----------------

Servo myservo;

#define TRIG 5

#define ECHO 18

#define SERVOPIN 4

// ---------------- Scan Speed Configuration ----------------

const float DIST_MIN = 5.0;    // cm - closest range (maximum speed)

const float DIST_MAX = 50.0;   // cm - farthest range (minimum speed)

const int MIN_DELAY = 5;       // ms per step at closest range (fastest)

const int MAX_DELAY = 40;      // ms per step at farthest range (slowest)

const int STEP_SIZE = 5;       // servo increment in degrees

// ---------------- Live State ----------------

long currentDistance = 0;

int currentDelay = MAX_DELAY;

String speedStatus = "SLOW";

// Waits without freezing the web page

void waitAndServe(int ms) {

unsigned long start = millis();

while (millis() - start < ms) {

server.handleClient();

delay(1);

}

}

// Measures distance using the HC-SR04

long getDistance() {

digitalWrite(TRIG, LOW);

delayMicroseconds(2);

digitalWrite(TRIG, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG, LOW);

long duration = pulseIn(ECHO, HIGH, 30000);  // 30 ms timeout

if (duration == 0) return DIST_MAX;          // no echo: treat as out of range

return duration * (0.034 / 2);

}

// Turns the distance into a speed (delay per step)

int calculateDelay(long distance) {

float percentage = (float)(distance - DIST_MIN) / (DIST_MAX - DIST_MIN);

if (percentage > 1.0) percentage = 1.0;

if (percentage < 0.0) percentage = 0.0;

return MIN_DELAY + (percentage * (MAX_DELAY - MIN_DELAY));

}

// Builds the web page

void handleRoot() {

String html = "<!DOCTYPE html><html><head><meta http-equiv='refresh' content='1'>";

html += "<title>Radar Scanner</title>";

html += "<style>body{font-family: Arial; text-align: center; margin-top: 50px;";

if (currentDistance < 15) {

html += "background-color: #f8d7da; color: #721c24;";

} else {

html += "background-color: #d4edda; color: #155724;";

}

html += "}</style></head><body>";

html += "<h1>Ultrasonic Radar Scanner</h1>";

html += "<h2>Distance: " + String(currentDistance) + " cm</h2>";

html += "<h2>Servo Speed: " + speedStatus + "</h2>";

html += "<h3>Step Delay: " + String(currentDelay) + " ms</h3>";

if (currentDistance < 15) {

html += "<h2 style='color:red;'>OBJECT CLOSE - SCANNING FAST</h2>";

}

html += "</body></html>";

server.send(200, "text/html", html);

}

void setup() {

Serial.begin(115200);

pinMode(TRIG, OUTPUT);

pinMode(ECHO, INPUT);

ESP32PWM::allocateTimer(0);

ESP32PWM::allocateTimer(1);

myservo.setPeriodHertz(50);

myservo.attach(SERVOPIN, 500, 2400);

myservo.write(0);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP address: ");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.begin();

Serial.println("Radar Ready.");

}

// One scan step: move, measure, set speed, wait

void doStep(int angle) {

myservo.write(angle);

currentDistance = getDistance();

currentDelay = calculateDelay(currentDistance);

if (currentDelay <= 12)      speedStatus = "VERY FAST";

else if (currentDelay <= 25) speedStatus = "MEDIUM";

else                         speedStatus = "SLOW";

Serial.print("Angle: ");

Serial.print(angle);

Serial.print(" | Distance: ");

Serial.print(currentDistance);

Serial.print(" cm | Delay: ");

Serial.print(currentDelay);

Serial.print(" ms | Speed: ");

Serial.println(speedStatus);

waitAndServe(currentDelay);

}

void loop() {

server.handleClient();

// Forward sweep

for (int angle = 0; angle <= 180; angle += STEP_SIZE) {

doStep(angle);

}

// Return sweep

for (int angle = 180; angle >= 0; angle -= STEP_SIZE) {

doStep(angle);

}

}

10. What Each Part of the Code Does

- getDistance() sends the sound pulse and works out the distance in centimetres. If no echo comes back in 30 ms, it just says the object is far away.

- calculateDelay() turns the distance into a waiting time. Closer means a smaller wait, which means a faster sweep.

- waitAndServe() is a special waiting function. A normal delay() would freeze the board and the web page would stop working. This one waits but keeps the web page alive.

- doStep() does one step of the scan: move the servo, measure the distance, set the speed, then wait. Both sweeps use it, so the code is written once and reused.

- handleRoot() builds the web page and turns it red when an object is close.

- loop() runs two for loops: one sweeps 0 to 180, the other sweeps 180 back to 0. Together they make the radar motion.

11. Settings You Can Change

| Setting | What it does |
| --- | --- |
| DIST_MIN = 5.0 | Distance treated as closest (fastest speed) |
| DIST_MAX = 50.0 | Distance treated as farthest (slowest speed) |
| MIN_DELAY = 5 | Wait at closest range. Lower means faster. |
| MAX_DELAY = 40 | Wait at farthest range. Higher means slower. |
| STEP_SIZE = 5 | How many degrees the servo moves each step |

Watch the Serial Monitor (set it to 115200) and change these numbers until the speed feels right.

12. What You Will See

- The Serial Monitor shows the angle, distance, delay, and speed for every step.

- Open the IP address shown in the Serial Monitor in a browser to see the live page.

- The servo sweeps back and forth like a radar.

- Move your hand closer and the sweep clearly speeds up. Move it away and it slows down.

- When something is closer than 15 cm, the web page turns red with a warning.

13. Where This Can Be Used

- Obstacle detection for robots

- Radar and area scanning demos

- Parking assistance systems

- Security scanning that reacts faster when someone is near

14. If Something Does Not Work

- Distance always shows 0 or a strange number? Check TRIG is on GPIO 5 and ECHO on GPIO 18.

- ESP32 keeps restarting when the servo moves? That is a power problem. Use a separate 5V supply for the servo and join all the grounds.

- Servo shakes or moves randomly? Check that all the grounds are connected together.

- Nothing on the Serial Monitor? Set the Serial Monitor speed to 115200.

- Web page will not open? Check the IP address, and make sure your phone is on the same Wi-Fi. The ESP32 only works on 2.4 GHz Wi-Fi, not 5 GHz. If college Wi-Fi does not work, use a phone hotspot.

- Speed does not change? Check the Serial Monitor to see if the distance readings are changing at all.