# RGB LED Slider Control via Blynk IoT

## Objective

Enable real-time control of an RGB LED using three sliders in the Blynk IoT dashboard.

## Components Required

- ESP32 development board

- Common‑cathode RGB LED with resistors

- Breadboard

- Jumper wires

## Circuit Connections

- Red LED pin → GPIO 25 (220 Ω)

- Green LED pin → GPIO 26 (220 Ω)

- Blue LED pin → GPIO 27 (220 Ω)

- Common Cathode → GND

## Software Requirements

- Arduino IDE

- ESP32 board support installed

- Latest Blynk IoT library (2025)

## Functional Overview

ESP32 connects to Blynk Cloud; slider callbacks set PWM on RGB channels; LED color changes in real time.

## Source Code

#define BLYNK_TEMPLATE_ID "TMPL3LGxyG3Jr"

#define BLYNK_TEMPLATE_NAME "RGB LED Slider"

#define BLYNK_AUTH_TOKEN "-gJzouWCyPnJHeJpmfuOerKbgZiq1BLF"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

//─── Wi-Fi & Blynk credentials ─────────────────────────────

char ssid[]     = "NSUT_WIFI";

char pass[]     = "";

//─── RGB LED pins & PWM channels ───────────────────────────

#define RED_PIN 25

#define GREEN_PIN 26

#define BLUE_PIN 27

// PWM settings: 1 kHz frequency, 8-bit resolution (0–255)

const int PWM_FREQ      = 1000;

const int PWM_RES_BITS  = 8;

void setup() {

Serial.begin(115200);

// Attach each channel to its pin

ledcAttach(RED_PIN,   PWM_FREQ, PWM_RES_BITS);

ledcAttach(GREEN_PIN, PWM_FREQ, PWM_RES_BITS);

ledcAttach(BLUE_PIN,  PWM_FREQ, PWM_RES_BITS);

// Connect to Blynk

Serial.println("Connecting to Blynk...");

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

Serial.println("\nBlynk connected");

}

// Handler for slider on V1 → Red

BLYNK_WRITE(V1) {

int value = param.asInt();          // 0–255 from slider

ledcWrite(RED_PIN, value);           // Set PWM duty

Serial.printf("Red = %d\n", value);

}

// Handler for slider on V2 → Green

BLYNK_WRITE(V2) {

int value = param.asInt();

ledcWrite(GREEN_PIN, value);

Serial.printf("Green = %d\n", value);

}

// Handler for slider on V3 → Blue

BLYNK_WRITE(V3) {

int value = param.asInt();

ledcWrite(BLUE_PIN, value);

Serial.printf("Blue = %d\n", value);

}

void loop() {

Blynk.run();  // Process Blynk events

}

## How It Works

- LEDC PWM channels control LED brightness

- Blynk WRITE handlers receive slider values and update the LED

- Changes reflect instantly.

## Output Example

On Blynk dashboard:

Three sliders for Red, Green, Blue

LED shows the chosen color immediately.

## Applications

- IoT lighting control

- Mood lamps and ambient lighting

- Learning hardware-to-web interaction

## Troubleshooting Tips

- Verify LED type (cathode vs anode) and wiring.

- Invert PWM values for common-anode LEDs.

- Check Blynk auth token and network settings.