Distance Detector using ESP32 and HC-SR04T

1. Objective

To develop a distance detection system using the ESP32 and HC-SR04T ultrasonic sensor.
The system measures the distance of any object in front of the sensor and displays it on a web server hosted by the ESP32.
It also triggers a buzzer and an LED if the object is too close (e.g., less than 20 cm).

2. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 Dev Board | 1 |
| HC-SR04T Ultrasonic Sensor | 1 |
| LED | 1 |
| Buzzer | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 (optional) |
| Jumper Wires | As needed |
| Micro USB Cable | 1 |

3. Circuit Connections

Connections:

| Component | ESP32 Pin |
| --- | --- |
| HC-SR04T TRIG | GPIO 5 |
| HC-SR04T ECHO | GPIO 18 |
| Buzzer | GPIO 19 |
| LED (+) | GPIO 4 (via 220Ω resistor) |
| All VCCs | VIN (5V) from ESP32 |
| All GNDs | GND |

4. Software Requirements

- Arduino IDE

- ESP32 Board Package (via Board Manager)

- WebServer library (included in ESP32 core)

- USB drivers for ESP32 (CH340 or CP210x depending on your board)

5. Functional Overview

- ESP32 connects to a Wi-Fi network (NSUT_WIFI).

- HC-SR04T continuously measures the distance of nearby objects.

- If the object is closer than 20 cm, the buzzer and LED are activated.

- A simple web server runs on the ESP32, displaying the live distance and alert messages.

- Webpage auto-refreshes every second to update the reading.

6. Source Code

🔸 Main Code

#define TRIG_PIN 5

#define ECHO_PIN 18

#define BUZZER_PIN 19

#define LED_PIN 4

#define DISTANCE_THRESHOLD 20

long getDistance() {

digitalWrite(TRIG_PIN, LOW); delayMicroseconds(4);

digitalWrite(TRIG_PIN, HIGH); delayMicroseconds(20);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 60000);

long dist = duration * 0.034 / 2;

return (dist <= 0 || dist > 400) ? -1 : dist;

}

void loop() {

long distance = getDistance();

if (distance > 0 && distance < DISTANCE_THRESHOLD) {

digitalWrite(BUZZER_PIN, HIGH);

digitalWrite(LED_PIN, HIGH);

} else {

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(LED_PIN, LOW);

}

server.handleClient();

delay(300);

}

7. How It Works

setup()

- Initializes serial communication

- Sets pin modes for TRIG, ECHO, BUZZER, LED

- Connects to Wi-Fi (NSUT_WIFI)

- Starts the web server

loop()

- Calls getDistance() to get the object distance from the sensor

- Validates that the distance is within the range (2 to 400 cm)

- If distance < 20 cm → turns on buzzer and LED

- Else → turns off buzzer and LED

- The webpage auto-refreshes every second to display updated distance

- server.handleClient() keeps the web server alive and responsive

8. Output Example

- Distance: 15 cm → Buzzer ON, LED ON → Web shows ⚠️ Object too close!

- Distance: 30 cm → Buzzer OFF, LED OFF → Web shows ✅ Safe distance

- Invalid reading → Web shows "-- No reading"

9. Applications

- Obstacle detection for robots

- Smart trash bins

- Parking assistance systems

- Entry/exit alert systems

- Proximity-based alarms

10. Enhancements

- Web UI upgrade: Add a progress bar or color indicator for distance.

- Adjustable threshold: Allow user to set buzzer trigger distance from the web.

- Beeping buzzer: Make buzzer beep intermittently instead of staying constantly ON.

- Wi-Fi Access Point Mode: Enable the ESP32 to host its own Wi-Fi network (no external router needed).

- Data logging: Store distance readings in memory or send to cloud using Firebase/MQTT.

- OLED/7-segment fallback: Option to display distance on a screen if network is not available.

- Power optimization: Add deep sleep when no movement is detected.

- Mobile App: Build an Android app to receive alerts from the ESP32.

11. Troubleshooting Tips

| Problem | Solution |
| --- | --- |
| Web page not loading | Use correct IP shown in Serial Monitor |
| Reading always 0 or 682 cm | Use VIN for 5V, check sensor wiring |
| Buzzer stays ON even when no object | Object may be constantly close or noisy sensor |
| Upload fails | Disconnect sensor pins during upload |
| Sensor not detecting anything | Ensure GND is common and power is stable (5V) |
| Distance fluctuates too much | Use better power supply or add median filtering |
| Serial Monitor shows nothing | Check baud rate (115200) and selected COM port |
| Upload freezes at “Connecting...” | Hold BOOT button on ESP32 while uploading |

12. Conclusion

This project demonstrates a practical and low-cost solution for real-time object distance detection using the ESP32 and HC-SR04T ultrasonic sensor. By integrating a buzzer, LED, and a web-based interface, it provides immediate feedback and remote monitoring of nearby obstacles.

The system is compact, reliable, and can be easily adapted for use in automation, robotics, smart bins, entry detection, and other real-world applications. With potential enhancements like cloud connectivity and mobile alerts, it can serve as a strong foundation for more advanced IoT-based proximity systems.