Joystick-Controlled Web Game using ESP32

1. Aim

To make a small game that you can play in a web browser and control using a joystick. The joystick is connected to an ESP32 board. When you move the joystick, the player in the game moves too. The joystick button is used to start the game again after it ends.

2. What is an ESP32? (Quick Note)

An ESP32 is a small computer board, a little like an Arduino, but with one big extra power: it can connect to Wi-Fi. This means it can act like a tiny website. Your phone or laptop can open that website in a browser and play the game. That is why we use it here.

3. Parts Needed

| Part | How Many |
| --- | --- |
| ESP32 Board | 1 |
| Joystick Module | 1 |
| Jumper Wires | 1 set |
| Breadboard (optional) | 1 |
| USB Cable | 1 |

4. How to Connect the Wires

The joystick has a few pins. Connect them to the ESP32 like this:

| Joystick Pin | Connect to ESP32 |
| --- | --- |
| VCC (power) | 3.3V |
| GND (ground) | GND |
| VRx (left-right) | GPIO 34 |
| SW (button) | GPIO 27 |

Simple tip: VCC is power, GND is ground, VRx tells the board how far left or right you moved, and SW is the button you press. In this game we only use left-right movement (VRx), so we do not need the up-down pin.

5. Software Needed

- Arduino IDE (the app where you write and upload the code)

- ESP32 board package added inside Arduino IDE (this teaches the app about the ESP32)

- A web browser like Chrome to play the game

- A USB cable to connect the board to your computer

6. How the Project Works (Simple Idea)

Think of the ESP32 as a tiny website. Here is the full idea in easy steps:

- The joystick tells the ESP32 how far you have moved it left or right.

- The ESP32 joins your Wi-Fi and becomes a small website.

- You open that website on your phone or laptop, and the game appears.

- The game keeps asking the ESP32, "Where is the joystick now?" many times every second.

- Whatever the joystick says, the player in the game moves to match it.

- Red blocks fall from the top. You move left and right to dodge them.

- If a block hits you, the game ends. Press the joystick button to play again.

7. The Code

Code for the ESP32

#include <WiFi.h>

#include <WebServer.h>

// ---------------- WiFi ----------------

const char* ssid = "NSUT_WIFI";

const char* password = "";

// ---------------- Joystick ----------------

const int joyX = 34;

const int joySW = 27;

WebServer server(80);

// Sends the joystick reading to the web page

void handleJoystick() {

int x = analogRead(joyX);

int sw = digitalRead(joySW);

String json = "{\"x\":";

json += x;

json += ",\"sw\":";

json += sw;

json += "}";

server.send(200, "application/json", json);

}

// Sends the game page to the browser

void handleRoot() {

server.send(200, "text/html", R"rawliteral(

... the game page (HTML + JavaScript) goes here ...

)rawliteral");

}

void setup() {

Serial.begin(115200);

pinMode(joySW, INPUT_PULLUP);

WiFi.begin(ssid, password);

Serial.print("Connecting");

int tries = 0;

while (WiFi.status() != WL_CONNECTED && tries < 30) {

delay(500);

Serial.print(".");

tries++;

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println();

Serial.println("Connected!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

} else {

Serial.println();

Serial.println("WiFi Failed!");

return;

}

server.on("/", handleRoot);

server.on("/data", handleJoystick);

server.begin();

Serial.println("Server Started");

}

void loop() {

server.handleClient();

}

Note: The game screen (the coloured blocks, the score, and the movement) is made using HTML and JavaScript. That part is sent to the browser by the handleRoot() section. It has been kept short here so the main ESP32 code is easy to read.

8. What Each Part of the Code Does

- The top lines load the Wi-Fi and web server tools so the ESP32 can act like a website.

- joyX = 34 and joySW = 27 tell the code which pins the joystick is plugged into.

- handleJoystick() reads the joystick and sends the numbers to the game page.

- handleRoot() sends the actual game (the picture you see and play).

- setup() runs once at the start: it joins Wi-Fi and starts the website. If Wi-Fi does not connect, it shows "WiFi Failed!".

- loop() runs again and again: it keeps the website working so the game stays live.

9. What You Will See

- After you upload the code, the Serial Monitor shows the word "Connected!" and an IP address (something like 192.168.1.5).

- Type that IP address into a browser on the same Wi-Fi. The game opens.

- Move the joystick left and right. The green player block moves with it.

- Red blocks fall down and get faster as your score goes up.

- If a block hits you, "GAME OVER" shows. Press the joystick button to restart.

10. Where This Can Be Used

- Shows how a joystick can control something over Wi-Fi

- Can be turned into a wireless game controller

- A good, fun project to learn IoT and coding basics

- Can be changed later to control other things, not just games

11. If Something Does Not Work

- Player does not move? Check that VRx is on GPIO 34 and the button on GPIO 27.

- Still no movement? Upload a small test that just prints the joystick number, to check the wiring.

- Make sure your phone or laptop is on the SAME Wi-Fi as the ESP32.

- The ESP32 only works on 2.4 GHz Wi-Fi, not 5 GHz. College Wi-Fi with a login page often does not work. If stuck, use a phone hotspot.

- Game does not open? Check you typed the IP address correctly in the browser.

- Give power to the joystick from 3.3V, not 5V, to keep the board safe.