# Ambient Light Reactive RGB Lighting System using ESP32

## 1. Objective

To design an ambient lighting system using an ESP32, an LDR (Light Dependent Resistor) sensor, and an RGB LED module. The ESP32 continuously monitors the surrounding light intensity and automatically controls the RGB LED. When the environment becomes dark, the RGB LED glows with a predefined ambient color, while it remains turned off during bright conditions.

## 2. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 Dev Board | 1 |
| LDR Sensor Module (Digital Output) | 1 |
| RGB LED Module (or RGB LED) | 1 |
| Breadboard | 1 |
| Jumper Wires | 1 Set |

## 3. Circuit Connections

### LDR Sensor Connections

| LDR Pin | ESP32 GPIO |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| OUT | GPIO 34 |

### RGB LED Connections

| RGB Pin | ESP32 GPIO |
| --- | --- |
| Red | GPIO 25 |
| Green | GPIO 26 |
| Blue | GPIO 27 |
| GND/Common | GND |

## 4. Software Requirements

- Arduino IDE (Latest Version)

- ESP32 Board Package installed in Arduino IDE

- USB Cable for ESP32 Programming

- Serial Monitor (115200 baud)

## 5. Functional Overview

The LDR sensor continuously detects the ambient light level and provides a digital output to the ESP32.

- If the surroundings are dark, the ESP32 turns on the RGB LED with a soft ambient cyan-blue color by adjusting the Red, Green, and Blue brightness levels.

- If the surroundings are bright, the ESP32 switches the RGB LED off to conserve power.

The current operating status is also displayed on the Arduino Serial Monitor.

## 6. Source Code

#include <WiFi.h>

#include <WebServer.h>

// Wi-Fi Credentials

const char* ssid = "NSUT_WIFI";

const char* password = "";

WebServer server(80);

// Pin Definitions

const int LDR_DIGITAL_PIN = 34; // Connected to 'OUT' pin on your module

const int RED_PIN = 25;         // RGB Module Red

const int GREEN_PIN = 26;       // RGB Module Green

const int BLUE_PIN = 27;        // RGB Module Blue

String currentStatus = "Unknown";

void handleRoot() {

String html = "<!DOCTYPE html><html><head><meta http-equiv='refresh' content='2'>";

html += "<title>Ambient Light Reactor</title>";

html += "<style>body{font-family: Arial; text-align: center; margin-top: 50px;}</style></head><body>";

html += "<h1>Ambient Light Reactor Status</h1>";

html += "<h2>Current State: " + currentStatus + "</h2>";

html += "</body></html>";

server.send(200, "text/html", html);

}

void setup() {

Serial.begin(115200);

pinMode(LDR_DIGITAL_PIN, INPUT);

pinMode(RED_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

pinMode(BLUE_PIN, OUTPUT);

// Connect to Wi-Fi

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP address: ");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.begin();

}

void loop() {

server.handleClient();

static int lastState = -1;

// Read the digital state from the 'OUT' pin

int isDark = digitalRead(LDR_DIGITAL_PIN);

if (isDark != lastState) {

if (isDark == HIGH) {

// ROOM IS DARK: Turn on a cozy ambient color (e.g., Warm Cyan/Blue)

analogWrite(RED_PIN, 100);

analogWrite(GREEN_PIN, 150);

analogWrite(BLUE_PIN, 255);

currentStatus = "Dark -> Ambient Reactor ON";

Serial.println("Status: Dark -> Ambient Reactor ON");

} else {

// ROOM IS BRIGHT: Turn off the RGB reactor

analogWrite(RED_PIN, 0);

analogWrite(GREEN_PIN, 0);

analogWrite(BLUE_PIN, 0);

currentStatus = "Bright -> Ambient Reactor OFF";

Serial.println("Status: Bright -> Ambient Reactor OFF");

}

lastState = isDark;

}

}

# 7. How It Works

- The ESP32 initializes the LDR sensor as a digital input and the RGB LED pins as outputs.

- The LDR continuously senses the surrounding light intensity.

- When darkness is detected, the LDR module outputs a HIGH signal.

- The ESP32 reads this signal and turns on the RGB LED with a predefined ambient color using PWM values.

- When sufficient light is detected, the LDR outputs LOW, causing the ESP32 to switch off the RGB LED.

- The current operating state ("Ambient Reactor ON" or "Ambient Reactor OFF") is displayed on the Serial Monitor every 200 milliseconds.

# 8. Output Example

### In Bright Environment

Status: Bright -> Ambient Reactor OFF

Status: Bright -> Ambient Reactor OFF

Status: Bright -> Ambient Reactor OFF

RGB LED remains OFF.

### In Dark Environment

Status: Dark -> Ambient Reactor ON

Status: Dark -> Ambient Reactor ON

Status: Dark -> Ambient Reactor ON

RGB LED glows with a cyan-blue ambient light.

# 9. Applications

- Automatic room ambient lighting

- Smart home lighting systems

- Night lamps

- Corridor and staircase lighting

- Energy-efficient lighting control

- Light-sensitive decorative lighting

- IoT-based home automation projects

- Educational embedded systems demonstrations

# 10. Troubleshooting Tips

- Ensure the LDR module is powered using 3.3V to maintain compatibility with the ESP32.

- Verify that the LDR module's OUT pin is connected to GPIO 34.

- Confirm that the RGB LED pins are correctly connected to GPIO 25, GPIO 26, and GPIO 27.

- If the RGB LED does not glow, verify the LED type (common anode or common cathode) and adjust the logic accordingly.

- Open the Serial Monitor at 115200 baud to monitor sensor status messages.

- If the sensor readings do not change, adjust the sensitivity potentiometer on the LDR module until it reliably detects light and darkness.

- Ensure all ground (GND) connections are common between the ESP32, LDR module, and RGB LED.

This documentation matches the style and structure of your previous ESP32 project, making it suitable for inclusion in your IoT project report.