# ESP32 Clap Detector Using Analog Sound Sensor

## Objective

To design and implement a clap detector using an ESP32 and an analog sound sensor. The ESP32 reads the analog output of the microphone and detects a clap whenever the sound level exceeds a predefined threshold.

## Theory

A sound sensor converts sound into an analog voltage. The ESP32 reads this voltage using its 12-bit ADC (0-4095). When the measured value exceeds a preset threshold, a clap is detected.

## Components Required

- ESP32 Development Board
- Analog Sound Sensor Module
- Jumper Wires
- USB Cable
- Breadboard (optional)

## Circuit Connections

VCC (+) -> 3.3V
GND (-) -> GND
AO (A) -> GPIO 34
DO (D) -> Not Connected

## Program

#include <WiFi.h>

#include <WebServer.h>

const char* ssid = "NSUT_WIFI";

const char* password = "";

WebServer server(80);

const int micPin = 34;      // AO connected to GPIO34

const int threshold = 0;

int clapCount = 0;

void handleRoot() {

String html = "<!DOCTYPE html><html><head><meta http-equiv='refresh' content='2'>";

html += "<title>Clap Detector</title>";

html += "<style>body{font-family: Arial; text-align: center; margin-top: 50px;}</style></head><body>";

html += "<h1>Clap Detector Status</h1>";

html += "<h2>Total Claps Detected: " + String(clapCount) + "</h2>";

html += "</body></html>";

server.send(200, "text/html", html);

}

void setup() {

Serial.begin(115200);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP address: ");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.begin();

}

void loop() {

server.handleClient();

int sound = analogRead(micPin);

static unsigned long lastPrint = 0;

if (millis() - lastPrint > 500) {

Serial.println(sound);    // View the analog value periodically

lastPrint = millis();

}

if (sound > threshold) {

Serial.println("Clap detected!");

clapCount++;

// non-blocking delay

unsigned long start = millis();

while (millis() - start < 300) { server.handleClient(); delay(1); }

}

}

## Working Principle

1. Capture sound.
2. Read analog value.
3. Display value on Serial Monitor.
4. Compare with threshold.
5. Print 'Clap detected!' when threshold is crossed.

## Procedure

1. Make the connections.
2. Upload the code.
3. Open Serial Monitor (115200 baud).
4. Observe readings.
5. Clap near the sensor.
6. Adjust threshold if required.

## Expected Output

Quiet:
1500
1498

Clap:
2870
Clap detected!

## Applications

Smart home automation, clap-operated switches, security systems, noise monitoring, IoT projects.

## Result

The ESP32 successfully detects a clap by comparing the analog sound level with a predefined threshold.

## Conclusion

The project demonstrates clap detection using the ESP32 and an analog sound sensor with adjustable software threshold.