# Instant Earthquake Alarm System using ESP32

## 1. Objective

To develop a real-time earthquake detection and warning system using an ESP32, a tilt/vibration sensor, and a buzzer. The ESP32 continuously monitors vibrations through the tilt sensor and immediately activates an alarm whenever shaking is detected, providing an instant warning with virtually zero response delay.

# 2. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 Dev Board | 1 |
| Tilt Sensor / Vibration Sensor Module | 1 |
| Active Buzzer (or Passive Buzzer) | 1 |
| Breadboard | 1 |
| Jumper Wires | 1 Set |
| USB Cable | 1 |

# 3. Circuit Connections

### Tilt Sensor

| Tilt Sensor Pin | ESP32 GPIO |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| DO (Digital Output) | GPIO 14 |

### Buzzer

| Buzzer Pin | ESP32 GPIO |
| --- | --- |
| Signal (+) | GPIO 13 |
| GND (–) | GND |

Note: The project supports both Active and Passive buzzers. For a passive buzzer, the tone() function can be used to generate an audible alarm.

# 4. Software Requirements

- Arduino IDE (Latest Version)

- ESP32 Board Package installed in Arduino IDE

- USB Cable for ESP32 Programming

- Serial Monitor (115200 baud)

# 5. Functional Overview

The tilt sensor continuously detects vibrations or sudden movements. The ESP32 reads the digital output from the sensor in real time without introducing any software delays.

- Under normal conditions, the buzzer remains OFF.

- When shaking or vibration is detected, the sensor changes its output state.

- The ESP32 immediately activates the buzzer to generate an audible warning.

- As soon as the vibration stops, the alarm is instantly turned OFF.

The system is designed for instantaneous response, making it suitable for demonstration of earthquake detection principles and vibration monitoring.

# 6. Source Code

#include <WiFi.h>

#include <WebServer.h>

const char* ssid = "NSUT_WIFI";

const char* password = "";

WebServer server(80);

// Instant-Trigger Earthquake Alarm (Zero Delay)

const int TILT_PIN = 14;   // Pin connected to Tilt Sensor DO

const int BUZZER_PIN = 13; // Pin connected to Buzzer Signal/IO

// --- CALIBRATION CONFIGURATION ---

// If the buzzer sounds when everything is still, change this to HIGH

const int SHAKE_DETECTED_STATE = LOW;

String alarmStatus = "SAFE";

void handleRoot() {

String html = "<!DOCTYPE html><html><head><meta http-equiv='refresh' content='1'>";

html += "<title>Earthquake Detector</title>";

html += "<style>body{font-family: Arial; text-align: center; margin-top: 50px;";

if (alarmStatus == "ALARM") {

html += "background-color: red; color: white;";

} else {

html += "background-color: #d4edda; color: #155724;";

}

html += "}</style></head><body>";

html += "<h1>Earthquake Monitor</h1>";

html += "<h2>Status: " + alarmStatus + "</h2>";

html += "</body></html>";

server.send(200, "text/html", html);

}

void setup() {

Serial.begin(115200);

pinMode(TILT_PIN, INPUT_PULLUP);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP address: ");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.begin();

Serial.println("System Ready. Instant real-time monitoring active.");

}

void loop() {

server.handleClient();

// Read the sensor instantly with zero blocking delays

if (digitalRead(TILT_PIN) == SHAKE_DETECTED_STATE) {

alarmStatus = "ALARM";

// METHOD 1: For Active Buzzers (Continuous, solid tone)

digitalWrite(BUZZER_PIN, HIGH);

// METHOD 2: For Passive Buzzers (Deep "Faahh" lower pitch horn)

// If your buzzer sounds like a click instead of a tone, uncomment the line below

// and comment out the digitalWrite line above.

// tone(BUZZER_PIN, 350);

} else {

alarmStatus = "SAFE";

// Instantly turn off the sound the microsecond shaking stops

digitalWrite(BUZZER_PIN, LOW);

// noTone(BUZZER_PIN); // Uncomment this if you are using Method 2

}

}

# 7. How It Works

- The ESP32 initializes the tilt sensor as a digital input using the internal pull-up resistor.

- The buzzer is configured as a digital output and remains OFF during startup.

- The ESP32 continuously reads the sensor output inside the loop() function.

- If the sensor detects vibration (configured as LOW in the program), the ESP32 immediately activates the buzzer.

- If no vibration is detected, the buzzer is switched OFF instantly.

- Since the code contains no delay() function, the monitoring and alarm response are nearly instantaneous.

# 8. Output Example

### Normal Condition

System Ready. Instant real-time monitoring active.

Buzzer Status: OFF

### Earthquake / Vibration Detected

System Ready. Instant real-time monitoring active.

Earthquake/Vibration Detected!

Alarm Activated

Buzzer Status: ON (Continuous Alarm)

### Vibration Stops

Monitoring...

Alarm Deactivated

Buzzer Status: OFF

# 9. Applications

- Earthquake early-warning demonstration systems

- Building vibration monitoring

- Industrial machine vibration detection

- Security and tamper detection systems

- Smart disaster alert prototypes

- Educational IoT and embedded systems projects

- Laboratory vibration monitoring

- Structural health monitoring demonstrations

# 10. Troubleshooting Tips

- Ensure the Tilt Sensor DO pin is connected to GPIO 14.

- Verify that the buzzer signal pin is connected to GPIO 13.

- If the buzzer remains ON continuously, change the following line in the code:

const int SHAKE_DETECTED_STATE = LOW;

to

const int SHAKE_DETECTED_STATE = HIGH;

depending on your sensor module's logic.

- If using a Passive Buzzer, comment out:

digitalWrite(BUZZER_PIN, HIGH);

and uncomment:

tone(BUZZER_PIN, 350);

Also uncomment:

noTone(BUZZER_PIN);

when stopping the alarm.

- Ensure all GND connections are common between the ESP32, sensor, and buzzer.

- Open the Serial Monitor at 115200 baud to verify that the system has initialized correctly.

- If the sensor does not respond, check the wiring and gently tilt or shake the sensor to verify its operation.

This documentation follows the same format and style as your previous IoT project reports, ensuring consistency across all your ESP32-based experiments.