Laser Tripwire Alarm using ESP32

1. Aim

To build a simple security alarm. A laser beam is pointed at a light sensor (LDR). As long as the beam hits the sensor, everything is quiet. If a person walks through and blocks the beam, the sensor goes dark and the buzzer sounds an alarm. This works just like the laser tripwires seen in movies.

2. What is an ESP32? (Quick Note)

An ESP32 is a small computer board, a little like an Arduino. We use it to read the light sensor and to switch the buzzer on and off. It runs the code that decides when the alarm should sound.

3. What is an LDR?

LDR means Light Dependent Resistor. It is a small sensor whose value changes with light. In bright light it gives a HIGH number, and in darkness it gives a LOW number. So by reading this number, the ESP32 can tell if the laser beam is hitting the sensor or has been blocked.

4. Parts Needed

| Part | How Many |
| --- | --- |
| ESP32 Board | 1 |
| LDR Sensor Module | 1 |
| Laser Emitter Module | 1 |
| Buzzer (passive) | 1 |
| Jumper Wires | 1 set |
| USB Cable | 1 |

5. How to Connect the Wires

The LDR module has 4 pins. We use the AO (analog out) pin, because it gives us a number we can compare. Connect everything like this:

| Part / Pin | Connect to ESP32 |
| --- | --- |
| LDR - AO (analog out) | GPIO 34 |
| LDR - VCC (power) | 3.3V |
| LDR - GND (ground) | GND |
| LDR - DO (not used) | leave empty |
| Laser - Signal | GPIO 5 |
| Laser - VCC / GND | 3.3V / GND |
| Buzzer - Signal | GPIO 4 |
| Buzzer - VCC / GND | 3.3V / GND |

Important: point the laser straight at the LDR, a few centimetres away, so the beam lands on the sensor. This is what keeps the sensor "bright" until someone blocks the beam.

Safety: never point the laser at anyone's eyes. Aim it only at the sensor.

6. Software Needed

- Arduino IDE (the app to write and upload the code)

- ESP32 board package added inside Arduino IDE

- A USB cable to connect the board

7. How the Project Works (Simple Idea)

- The laser shines on the LDR, so the LDR reads a HIGH number (bright).

- The ESP32 keeps reading this number again and again.

- When someone walks through, they block the beam. The LDR goes dark and the number drops LOW.

- The code checks: if the number is below 1000, the beam is broken.

- If the beam is broken, the buzzer turns ON. If not, the buzzer stays OFF.

8. The Code

#include <WiFi.h>

#include <WebServer.h>

const char* ssid = "NSUT_WIFI";

const char* password = "";

WebServer server(80);

int LDR = 34;

int buzzer = 4;

int laser = 5;

// PWM settings

const int buzzerChannel = 0;

const int buzzerFreq = 2000;      // 2 kHz

const int buzzerResolution = 8;

String tripwireStatus = "INTACT";

void handleRoot() {

String html = "<!DOCTYPE html><html><head><meta http-equiv='refresh' content='1'>";

html += "<title>Laser Tripwire</title>";

html += "<style>body{font-family: Arial; text-align: center; margin-top: 50px;";

if (tripwireStatus == "BROKEN") {

html += "background-color: red; color: white;";

} else {

html += "background-color: #d4edda; color: #155724;";

}

html += "}</style></head><body>";

html += "<h1>Laser Tripwire Security</h1>";

html += "<h2>Status: " + tripwireStatus + "</h2>";

html += "</body></html>";

server.send(200, "text/html", html);

}

void setup() {

Serial.begin(115200);

pinMode(LDR, INPUT);

pinMode(laser, OUTPUT);

digitalWrite(laser, HIGH);

ledcAttach(buzzer, buzzerFreq, buzzerResolution);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP address: ");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.begin();

}

void loop() {

server.handleClient();

int value = analogRead(LDR);

static unsigned long lastPrint = 0;

if (millis() - lastPrint > 500) {

Serial.println(value);

lastPrint = millis();

}

if (value < 1000) {

tripwireStatus = "BROKEN";

ledcWrite(buzzer, 128);   // 50% duty cycle -> buzzer ON

} else {

tripwireStatus = "INTACT";

ledcWrite(buzzer, 0);     // buzzer OFF

}

}

9. What Each Part of the Code Does

- LDR = 34, buzzer = 4, laser = 5 tell the code which pin each part is connected to.

- digitalWrite(laser, HIGH) turns the laser on and keeps it on, like a steady beam.

- ledcAttach(...) gets the buzzer ready to make a sound. (On the ESP32, this is how we make a buzzer beep, instead of the Arduino way.)

- analogRead(LDR) reads the light level as a number from 0 to 4095.

- if (value < 1000) checks if it has gone dark. If yes, ledcWrite(buzzer, 128) sounds the alarm. If no, ledcWrite(buzzer, 0) keeps it silent.

- delay(50) makes the board wait a tiny moment before checking again.

10. Setting the Right Number (Calibration)

The number 1000 in the code is a guess. You should check the real numbers and adjust it:

- Open the Serial Monitor (set it to 115200).

- With the laser hitting the LDR, note the bright number (it will be high, maybe 3000-4000).

- Now block the beam and note the dark number (it will be much lower).

- Set the value in the code to somewhere between the two. Then the alarm triggers correctly.

11. What You Will See

- The Serial Monitor shows the light number changing all the time.

- Beam hitting the sensor: high number, buzzer silent.

- Beam blocked by a hand or person: number drops, buzzer sounds.

- Move away and let the beam hit again: buzzer stops.

12. Where This Can Be Used

- Home or room security alarm

- Detecting people crossing a doorway

- Counting objects passing on a line

- Simple burglar alarm projects

13. If Something Does Not Work

- Buzzer always ON? The beam may not be hitting the LDR. Aim the laser carefully, or raise the number (try 3000).

- Buzzer never sounds? Your number may be too low. Check the Serial values and set it between bright and dark.

- No numbers on the Serial Monitor? Set the Serial Monitor speed to 115200.

- Make sure the LDR's AO pin is on GPIO 34, not the DO pin.

- Give power to the LDR and laser from 3.3V, not 5V, to keep the board safe.