Touch-Activated Buzzer using ESP32

1. Objective

To develop an interactive buzzer system using ESP32 and KY-036 touch sensor that plays a custom melody when touched. The tune plays only during touch and resumes from the same note if interrupted.

2. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 Dev Board | 1 |
| KY-036 Touch Sensor | 1 |
| Active Buzzer | 1 |
| Breadboard | 1 |
| Jumper Wires | 3–4 |
| USB Cable (for ESP32) | 1 |
| Computer (for coding) | 1 |

3. Circuit Diagram

Connections:

| KY-036 Pin | ESP32 Pin |
| --- | --- |
| AO (Analog Output) | GPIO 34 |
| VCC | 3.3V |
| GND | GND |
| Buzzer (long pin +) | GPIO 25 |
| Buzzer (short pin −) | GND |

Tip: Use a 100Ω resistor in series with the buzzer to limit current (optional but safe).

4. Software Requirements

- Arduino IDE

- ESP32 Board Package (via Board Manager)

- USB Drivers for ESP32

- Basic C++/Arduino programming knowledge

5. Functional Overview

- ESP32 continuously reads analog input from the KY-036 touch sensor.

- When touch is detected (value below a set threshold), it plays a sequence of tones forming a melody.

- If the touch is removed, the buzzer stops instantly.

- When touched again, playback resumes from the last note.

- Once the full melody finishes, it restarts from the beginning on next touch.

6. Source Code

#include <WiFi.h>

#include <WebServer.h>

const char* ssid = "NSUT_WIFI";

const char* password = "";

WebServer server(80);

const int touchSensorPin = 34;   // KY-036 analog out

const int buzzerPin = 25;        // Active buzzer pin

int threshold = 2000;            // Adjust as needed

String touchStatus = "WAITING";

// === Luis Fonsi - Despacito (Chorus Hook) ===

int melody[] = {

// Des-pa-ci-to

587, 554, 494, 370,                      // D5, C#5, B4, F#4

// Pasito a pasito, suave suavecito

370, 370, 370, 370, 494, 494, 494, 494,  // F#4s then B4s

494, 440, 494, 392,                      // B4, A4, B4, G4

// Poquito a poquito, suave suavecito

392, 392, 392, 392, 494, 494, 494, 494,  // G4s then B4s

494, 554, 587, 440                       // B4, C#5, D5, A4

};

int noteDurations[] = {

// Des-pa-ci-to timing

4, 8, 4, 2,

// Pasito a pasito timing (Fast 16th notes)

16, 16, 16, 16, 16, 16, 16, 16,

8, 8, 8, 2,

// Poquito a poquito timing

16, 16, 16, 16, 16, 16, 16, 16,

8, 8, 8, 2

};

int noteIndex = 0;

void handleRoot() {

String html = "<!DOCTYPE html><html><head><meta http-equiv='refresh' content='1'>";

html += "<title>Melody Buzzer</title>";

html += "<style>body{font-family: Arial; text-align: center; margin-top: 50px;}</style></head><body>";

html += "<h1>IoT Doorbell</h1>";

html += "<h2>Touch Status: " + touchStatus + "</h2>";

html += "<br><a href=\"/play\"><button style=\"font-size:24px; padding:15px 30px;\">RING DOORBELL</button></a>";

html += "</body></html>";

server.send(200, "text/html", html);

}

bool webTrigger = false;

void handlePlay() {

webTrigger = true;

server.sendHeader("Location", "/");

server.send(303);

}

void setup() {

Serial.begin(115200);

pinMode(buzzerPin, OUTPUT);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP address: ");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.on("/play", handlePlay);

server.begin();

}

void playCurrentNote() {

if (noteIndex >= sizeof(melody) / sizeof(int)) {

noteIndex = 0;  // Restart riff

return;

}

int duration = 1290 / noteDurations[noteIndex];

tone(buzzerPin, melody[noteIndex]);

// Custom delay allowing handleClient to run

unsigned long start = millis();

while(millis() - start < duration) {

server.handleClient();

delay(1);

}

noTone(buzzerPin);

start = millis();

while(millis() - start < 30) {

server.handleClient();

delay(1);

}

noteIndex++;

}

void loop() {

server.handleClient();

int touchValue = analogRead(touchSensorPin);

static unsigned long lastPrint = 0;

if (millis() - lastPrint > 500) {

Serial.println(touchValue);

lastPrint = millis();

}

if (touchValue < threshold || webTrigger) {

touchStatus = (webTrigger) ? "WEB TRIGGERED (PLAYING)" : "TOUCHED (PLAYING)";

playCurrentNote();

if (noteIndex == 0) {

webTrigger = false; // Stop when song finishes

}

} else {

touchStatus = "WAITING";

noTone(buzzerPin);  // Silence if not touching

}

}

7. How It Works

- setup()
Initializes serial communication and configures the buzzer pin.

- loop()
Reads analog input from the touch sensor. If touched, it plays the next note using the playCurrentNote() function.

- playCurrentNote()
Plays one note from the melody array, waits for the duration, and then stops the buzzer. If the full melody has been played, it resets to the beginning.

8. Output Example

When touched:

- Buzzer plays: G+ A+ F+ G+, A# A+ F+ G+

- Pattern repeats 4 times

- Music stops if finger lifted

- Music resumes from same note on next touch

9. Applications

- Themed interactive DIY projects

- Fun buzzer alarms and doorbells

- Educational touch + sound projects

- Creative tech installations (parties, demos)

10. Enhancements (Future Scope)

- Multiple Melodies Support
Add button or second touch sensor to toggle between different songs (e.g., Happy Birthday, national anthem, Mario theme).

- Auto-Repeat Mode
Enable the melody to loop continuously while the sensor is being touched instead of note-by-note control.

- LED Visual Effects
Sync an RGB LED or LED strip to flash with the beat of the melody for a more dramatic effect.

- Bluetooth Control
Use ESP32’s built-in Bluetooth to trigger the melody remotely from a phone.

- Gesture-Based Playback
Use a gesture sensor instead of touch to play different notes with different gestures.

- Amplified Sound
Integrate a transistor or amplifier circuit with a larger speaker for louder sound output.

- Battery Power Option
Add a rechargeable battery module to make the project portable.

- Volume & Speed Control
Use potentiometers or rotary encoders to adjust the pitch speed or volume of the buzzer tones.

11. Troubleshooting Tips

| Issue | Solution |
| --- | --- |
| Buzzer not sounding | Check wiring; ensure you're using an active buzzer. |
| Touch not detected | Monitor analog values via Serial Monitor; adjust threshold. |
| Constant buzzing | Check if touch sensor is overly sensitive; raise threshold. |
| Melody not restarting | Ensure noteIndex is reset after all notes played. |
| ESP32 not uploading code | Check COM port and board selection in Arduino IDE. |

12. Conclusion

This project successfully demonstrates how an ESP32 microcontroller and a KY-036 touch sensor can be used to create an interactive and entertaining system that plays a melody through a buzzer. The touch-based activation provides a fun and intuitive interface, while the ability to resume playback from the last note adds a unique layer of interactivity.

The project not only showcases basic interfacing of sensors and actuators with ESP32 but also serves as a great introduction to timing, analog sensing, and sound generation in embedded systems. It is ideal for educational demonstrations, DIY musical toys, and creative projects that aim to combine technology with fun.