# Temperature-Controlled Variable Speed Servo Fan using ESP32

## 1. Objective

To design a smart temperature-controlled servo fan using an ESP32, a DHT11 temperature sensor, and a servo motor. The ESP32 continuously monitors the ambient temperature and automatically adjusts the sweeping speed of the servo-based fan according to the measured temperature. Higher temperatures result in faster oscillation, while the fan remains idle when the temperature is below a predefined threshold.

# 2. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 Dev Board | 1 |
| DHT11 Temperature Sensor | 1 |
| Servo Motor (SG90/MG90S) | 1 |
| Breadboard | 1 |
| Jumper Wires | 1 Set |
| USB Cable | 1 |

# 3. Circuit Connections

### DHT11 Sensor

| DHT11 Pin | ESP32 GPIO |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| DATA | GPIO 4 |

### Servo Motor

| Servo Pin | ESP32 GPIO |
| --- | --- |
| Signal | GPIO 18 |
| VCC | 5V (Recommended External Supply) |
| GND | GND |

Note: Always connect the servo's GND with the ESP32 GND. For reliable operation, power the servo from an external 5V supply instead of the ESP32.

# 4. Software Requirements

- Arduino IDE (Latest Version)

- ESP32 Board Package installed in Arduino IDE

- DHT Sensor Library

- ESP32Servo Library

- USB Cable for ESP32 Programming

- Serial Monitor (115200 baud)

# 5. Functional Overview

The DHT11 sensor continuously measures the surrounding temperature and sends the readings to the ESP32.

The ESP32 compares the measured temperature against two predefined thresholds:

- 25°C → Fan begins oscillating slowly.

- 35°C → Fan reaches maximum sweeping speed.

Instead of changing the servo angle randomly, the ESP32 continuously sweeps the servo from 0° to 180° and back, simulating a rotating fan head. As the temperature rises, the delay between successive servo movements decreases, resulting in faster oscillation.

The estimated fan speed (RPM) is also calculated mathematically and displayed on the Serial Monitor along with the temperature.

# 6. Source Code

#include <DHT.h>

#include <ESP32Servo.h>

#include <WiFi.h>

#include <WebServer.h>

const char* ssid = "NSUT_WIFI";

const char* password = "";

WebServer server(80);

#define DHTPIN 4          // GPIO 4 for DHT11 data

#define DHTTYPE DHT11

#define SERVOPIN 18       // GPIO 18 for Servo signal

DHT dht(DHTPIN, DHTTYPE);

Servo fanServo;

const float TEMP_MIN = 25.0;

const float TEMP_MAX = 35.0;

const int MAX_DELAY = 45;

const int MIN_DELAY = 8;

const int STEP_SIZE = 5;

float currentTemp = 0.0;

String fanStatus = "IDLE";

String warningMsg = "";

void handleRoot() {

String html = "<!DOCTYPE html><html><head><meta http-equiv='refresh' content='2'>";

html += "<title>Smart Fan</title>";

html += "<style>body{font-family: Arial; text-align: center; margin-top: 50px;}</style></head><body>";

html += "<h1>Temperature Controlled Fan</h1>";

html += "<h2>Current Temp: " + String(currentTemp, 1) + " &deg;C</h2>";

html += "<h2>Fan Status: " + fanStatus + "</h2>";

if (warningMsg != "") {

html += "<h2 style='color:red;'>" + warningMsg + "</h2>";

}

html += "</body></html>";

server.send(200, "text/html", html);

}

void setup() {

Serial.begin(115200);

dht.begin();

ESP32PWM::allocateTimer(0);

ESP32PWM::allocateTimer(1);

fanServo.setPeriodHertz(50);

fanServo.attach(SERVOPIN, 500, 2400);

fanServo.write(0);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected! IP address: ");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.begin();

}

void loop() {

server.handleClient();

float temp = dht.readTemperature();

if (isnan(temp)) {

// delay(2000); // Replacing blocking delay

unsigned long start = millis();

while(millis() - start < 2000) { server.handleClient(); delay(10); }

return;

}

currentTemp = temp;

if (temp >= TEMP_MAX) {

warningMsg = "WARNING: HIGH TEMPERATURE";

} else if (temp >= TEMP_MIN) {

warningMsg = "Fan is running to cool down.";

} else {

warningMsg = "";

}

if (temp >= TEMP_MIN) {

fanStatus = "ON";

float tempPercentage = (temp - TEMP_MIN) / (TEMP_MAX - TEMP_MIN);

if (tempPercentage > 1.0) tempPercentage = 1.0;

int sweepDelay = MAX_DELAY - (tempPercentage * (MAX_DELAY - MIN_DELAY));

float totalCycleTimeMs = (360 / STEP_SIZE) * 2 * sweepDelay;

float estimatedRPM = 60000.0 / totalCycleTimeMs;

Serial.print("Temp: ");

Serial.print(temp, 1);

Serial.print(" °C | Step Delay: ");

Serial.print(sweepDelay);

Serial.print("ms | Fan Speed: ");

Serial.print(estimatedRPM, 1);

Serial.println(" RPM");

for (int angle = 0; angle <= 180; angle += STEP_SIZE) {

fanServo.write(angle);

unsigned long start = millis();

while(millis() - start < sweepDelay) { server.handleClient(); delay(1); }

}

for (int angle = 180; angle >= 0; angle -= STEP_SIZE) {

fanServo.write(angle);

unsigned long start = millis();

while(millis() - start < sweepDelay) { server.handleClient(); delay(1); }

}

} else {

fanServo.write(0);

fanStatus = "IDLE";

float estimatedRPM = 0.0;

Serial.print("Temp: ");

Serial.print(temp, 1);

Serial.print(" °C | Fan Status: IDLE | Speed: ");

Serial.print(estimatedRPM, 1);

Serial.println(" RPM");

for(int i=0; i<20; i++){ server.handleClient(); delay(100); }

}

}

# 7. How It Works

- The ESP32 initializes the DHT11 sensor and servo motor.

- Every cycle, the ESP32 reads the ambient temperature from the DHT11.

- If the sensor reading fails, an error message is displayed on the Serial Monitor.

- When the temperature is below 25°C, the servo remains at its home position (0°), indicating the fan is idle.

- Once the temperature reaches 25°C or above, the ESP32 calculates a temperature percentage between 25°C and 35°C.

- This percentage is used to determine the delay between servo movements:

  - Lower temperature → Larger delay → Slower oscillation

  - Higher temperature → Smaller delay → Faster oscillation

- The servo sweeps smoothly from 0° to 180° and back to 0°, creating an oscillating fan motion.

- The ESP32 estimates the fan's equivalent RPM based on the sweep timing and displays it along with the temperature on the Serial Monitor.

# 8. Output Example

### Below 25°C

System Initialized: Variable Speed Servo Fan active.

Temp: 23.8 °C | Fan Status: IDLE | Speed: 0.0 RPM

Temp: 24.6 °C | Fan Status: IDLE | Speed: 0.0 RPM

Servo remains stationary at 0°.

### Between 25°C and 35°C

Temp: 27.4 °C | Step Delay: 36 ms | Fan Speed: 11.6 RPM

Temp: 30.2 °C | Step Delay: 24 ms | Fan Speed: 17.4 RPM

Temp: 34.8 °C | Step Delay: 9 ms | Fan Speed: 46.3 RPM

Servo continuously sweeps between 0° and 180°, with the oscillation speed increasing as the temperature rises.

# 9. Applications

- Smart temperature-controlled cooling systems

- Automatic room ventilation

- Miniature smart fan prototypes

- Greenhouse climate control

- IoT-based home automation

- Educational embedded systems projects

- Smart HVAC demonstrations

- Energy-efficient cooling systems

# 10. Troubleshooting Tips

- Ensure the DHT11 DATA pin is connected to GPIO 4.

- Verify that the servo signal pin is connected to GPIO 18.

- Power the servo using an external 5V supply if it jitters or resets the ESP32.

- Install the DHT Sensor and ESP32Servo libraries before compiling the code.

- Open the Serial Monitor at 115200 baud to view temperature, sweep delay, and estimated RPM.

- If the DHT11 returns invalid readings, check the sensor wiring and allow a few seconds after startup for stabilization.

- Ensure the servo's ground and ESP32 ground are connected together.

- If the servo does not move, verify that the ESP32 supports PWM on the selected GPIO and that the servo receives adequate power.

This documentation follows the same structure and style as your previous ESP32 project reports, making it consistent for inclusion in your IoT project documentation.