# IoT Smart Proximity Security System using ESP32

## 1. Objective

To design and deploy an intelligent, non-blocking **Proximity Security Monitor** using an ESP32 micro-controller and an HC-SR04 ultrasonic sensor. The system continually evaluates spatial distance parameters, calculating a multi-tier alert response across a local hardware array (an RGB LED indicator and an active buzzer alarm). Simultaneously, it hosts a beautiful, asynchronous web dashboard featuring glassmorphism design layouts, allowing real-time monitoring across a local network without causing page-load latency or system blocking.

---

## 2. Components Required

| Component | Quantity |

| --- | --- |

| **ESP32 Development Board** | 1 |

| **HC-SR04 Ultrasonic Sensor** | 1 |

| **RGB LED Module (Common Cathode)** | 1 |

| **Active Buzzer Module** | 1 |

| **220Ω Resistors** (For RGB Line Protection) | 3 |

| **Voltage Divider Resistor Pair** (1kΩ and 2kΩ) | 1 Set |

| **Breadboard** | 1 |

| **Jumper Wires** | As Needed |

| **Micro-USB Cable** | 1 |

---

## 3. Circuit Connections

### ⚠️ Pre-Assembled Hardware Guard

> The 5V signal generated by the HC-SR04 Echo line must pass through a pre-calculated **voltage divider** network before connecting to the ESP32 to shift the raw voltage down to a safe 3.3V operating threshold.

| Component Pin | ESP32 Target Pin | Notes / Function |

| --- | --- | --- |

| **HC-SR04 VCC** | **Vin (5V)** | Powers the acoustic transducers |

| **HC-SR04 TRIG** | **GPIO 5** | Outputs the initialization trigger pulses |

| **HC-SR04 ECHO** | **GPIO 18** | **Routed through the Voltage Divider** |

| **RGB Module R (Red)** | **GPIO 13** | Linked via 220Ω current-limiting resistor |

| **RGB Module G (Green)** | **GPIO 12** | Linked via 220Ω current-limiting resistor |

| **RGB Module B (Blue)** | **GPIO 14** | Linked via 220Ω current-limiting resistor |

| **Active Buzzer Signal** | **GPIO 27** | Controls the internal oscillation pattern |

| **All Module GNDs** | **GND Rail** | Common reference ground framework |

---

## 4. Software Requirements

* **Arduino IDE** (Configured for ESP32 Core Execution)

* **ESP32 Core Package Expansion** (Available via Board Manager)

* **WebServer Library** (Bundled within the core ESP32 environment framework)

---

## 5. Functional Overview

* **Network Initialization:** The ESP32 logs securely onto a local Wi-Fi router framework (`NSUT_WIFI`).

* **Spatial Evaluation Loop:** The sensor tracks nearby obstacles using ultrasonic Time-of-Flight pulses.

* **Multi-Tier Boundary Control Matrix:**

* **Safe Zone (`> 30 cm`):** The dashboard shifts green; the RGB LED shines Green; the buzzer remains silent.

* **Warning Zone (`15 cm to 30 cm`):** The dashboard shifts orange; the RGB LED projects an Orange/Yellow mix; the buzzer remains silent.

* **Critical Breach Zone (`< 15 cm`):** The dashboard flashes red; the RGB LED flashes Red in tandem with an active chirping alarm pattern.

* **Asynchronous Web Engine:** The device initializes a dual-endpoint local web framework. A visitor loading the core layout will hit a beautifully formatted responsive control canvas. The embedded script then performs asynchronous AJAX requests to a secondary `/data` endpoint every 250 milliseconds, repainting values smoothly without screen flicker.

---

## 6. Source Code

```cpp

#include <WiFi.h>

#include <WebServer.h>

// --- Wi-Fi Network Credentials ---

const char* ssid = "NSUT_WIFI";

const char* password = "";

// --- Hardware Pin Configurations ---

const int TRIG_PIN = 5;

const int ECHO_PIN = 18;

const int BUZZER_PIN = 27;

const int RED_PIN = 13;

const int GREEN_PIN = 12;

const int BLUE_PIN = 14;

// --- Distance Threshold Configurations (in cm) ---

const int SAFE_DISTANCE = 30;

const int WARNING_DISTANCE = 15;

// --- Global Variable State ---

long currentDistance = 0;

String safetyStatus = "Safe";

String dynamicColor = "#10b981";

WebServer server(80);

// --- Distance Calculation Engine ---

long getDistance() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 30000); // 30ms timeout prevents code hang

if (duration == 0) return 999;

long distance = duration * (0.034 / 2);

return distance;

}

// --- Asynchronous API Endpoint for Web Data ---

void handleReadings() {

String jsonPayload = "{";

jsonPayload += "\"distance\":" + String(currentDistance) + ",";

jsonPayload += "\"status\":\"" + safetyStatus + "\",";

jsonPayload += "\"color\":\"" + dynamicColor + "\"";

jsonPayload += "}";

server.send(200, "application/json", jsonPayload);

}

// --- HTML / CSS / JS UI Webpage Compilation ---

void handleRoot() {

String html = R"rawhtml(

<!DOCTYPE html>

<html lang='en'>

<head>

<meta charset='UTF-8'>

<meta name='viewport' content='width=device-width, initial-scale=1.0'>

<title>IoT Proximity Dashboard</title>

<style>

:root {

--bg-dark: #0f172a;

--card-bg: rgba(30, 41, 59, 0.7);

--text-main: #f8fafc;

--text-muted: #94a3b8;

}

* { box-sizing: border-box; margin: 0; padding: 0; font-family: '-apple-system', BlinkMacSystemFont, 'Segoe UI', Roboto, sans-serif; }

body {

background: radial-gradient(circle at top right, #1e1b4b, var(--bg-dark));

color: var(--text-main);

min-height: 100vh;

display: flex;

justify-content: center;

align-items: center;

padding: 20px;

}

.container {

width: 100%;

max-width: 480px;

background: var(--card-bg);

backdrop-filter: blur(16px);

border: 1px solid rgba(255, 255, 255, 0.1);

border-radius: 24px;

padding: 32px;

box-shadow: 0 20px 40px rgba(0,0,0,0.3);

text-align: center;

}

h1 { font-size: 1.8rem; font-weight: 700; margin-bottom: 8px; letter-spacing: -0.5px; }

.subtitle { color: var(--text-muted); font-size: 0.95rem; margin-bottom: 32px; }

.metric-card {

background: rgba(15, 23, 42, 0.5);

border-radius: 16px;

padding: 24px;

margin-bottom: 24px;

border: 1px solid rgba(255, 255, 255, 0.05);

}

.label { font-size: 0.85rem; text-transform: uppercase; letter-spacing: 1.5px; color: var(--text-muted); margin-bottom: 8px; }

.value { font-size: 3.5rem; font-weight: 800; line-height: 1; margin-bottom: 4px; transition: color 0.3s ease; }

.unit { font-size: 1.2rem; font-weight: 500; color: var(--text-muted); }

.status-badge {

display: inline-block;

padding: 8px 20px;

border-radius: 100px;

font-size: 0.9rem;

font-weight: 600;

letter-spacing: 0.5px;

text-transform: uppercase;

transition: all 0.3s ease;

}

.footer { margin-top: 32px; font-size: 0.8rem; color: var(--text-muted); }

</style>

</head>

<body>

<div class='container'>

<h1>Proximity Monitor</h1>

<p class='subtitle'>ESP32 Distance Detector System</p>

<div class='metric-card'>

<p class='label'>Live Distance</p>

<div class='value' id='distance-val'>-- <span class='unit'>cm</span></div>

</div>

<div class='status-badge' id='status-lbl' style='background: rgba(255,255,255,0.1); color: #fff;'>

Initializing...

</div>

<p class='footer'>Node MCU System Running Asynchronous Polling</p>

</div>

<script>

function updateDashboard() {

fetch('/data')

.then(response => response.json())

.then(data => {

const valEl = document.getElementById('distance-val');

const lblEl = document.getElementById('status-lbl');

valEl.innerHTML = data.distance + " <span class='unit'>cm</span>";

valEl.style.color = data.color;

lblEl.innerText = data.status;

lblEl.style.backgroundColor = data.color + "20";

lblEl.style.color = data.color;

lblEl.style.border = "1px solid " + data.color;

})

.catch(err => console.error("Error fetching IoT metrics:", err));

}

setInterval(updateDashboard, 250);

</script>

</body>

</html>

)rawhtml";

server.send(200, "text/html", html);

}

void setup() {

Serial.begin(115200);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(RED_PIN, OUTPUT);

pinMode(GREEN_PIN, OUTPUT);

pinMode(BLUE_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(RED_PIN, LOW);

digitalWrite(GREEN_PIN, LOW);

digitalWrite(BLUE_PIN, LOW);

WiFi.begin(ssid, password);

Serial.print("Connecting to Local Wi-Fi Network");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWi-Fi Connection Established!");

Serial.print("Local Network IP Target: http://");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.on("/data", handleReadings);

server.begin();

}

void loop() {

server.handleClient();

currentDistance = getDistance();

if (currentDistance > SAFE_DISTANCE) {

safetyStatus = "Clear / Safe";

dynamicColor = "#10b981";

digitalWrite(RED_PIN, LOW);

digitalWrite(GREEN_PIN, HIGH);

digitalWrite(BLUE_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

}

else if (currentDistance <= SAFE_DISTANCE && currentDistance > WARNING_DISTANCE) {

safetyStatus = "Warning Approaching";

dynamicColor = "#f59e0b";

digitalWrite(RED_PIN, HIGH);

digitalWrite(GREEN_PIN, HIGH);

digitalWrite(BLUE_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

}

else {

safetyStatus = "CRITICAL BREACH!";

dynamicColor = "#ef4444";

digitalWrite(GREEN_PIN, LOW);

digitalWrite(BLUE_PIN, LOW);

if ((millis() / 150) % 2 == 0) {

digitalWrite(RED_PIN, HIGH);

digitalWrite(BUZZER_PIN, HIGH);

} else {

digitalWrite(RED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

}

}

}

```

---

## 7. How It Works

### `setup()`

* Initializes terminal trace links at `115200` baud.

* Maps pin functionalities (`OUTPUT` for structural lines, `INPUT` for tracking echo vectors).

* Configures local router handshake routines, returning the unique target dashboard URL.

* Mounts internet listener portals for structural HTML packages and raw dynamic data vectors.

### `loop()`

* Executes web traffic resolution logic.

* Computes absolute spatial depth using ultrasonic time calculations.

* Filters data across boundaries to update state indicators:

* Over 30 cm resets alarms and illuminates the green hardware node.

* Between 15 and 30 cm mixes Red and Green channels to yield an Orange alert.

* Below 15 cm executes asynchronous blinking alerts without pausing system operations.

---

## 8. Output Example

* **Distance: `45 cm**` ➡️ RGB Green, Buzzer Silent ➡️ Dashboard displays: **`Clear / Safe`** with green accent styling.

* **Distance: `22 cm**` ➡️ RGB Orange/Yellow, Buzzer Silent ➡️ Dashboard displays: **`Warning Approaching`** with orange text tags.

* **Distance: `8 cm**` ➡️ RGB Pulsing Red, Buzzer Intermittent Beeps ➡️ Dashboard displays: **`CRITICAL BREACH!`** with an urgent crimson layout.

---

## 9. Applications

* Automotive backing safety layouts.

* Industrial safe-distance perimeter protection barriers.

* Modern warehouse facility asset organization.

* Touch-less entry system alert triggers.

---

## 10. Troubleshooting Tips

| Issue Identified | Resolution Actions |

| --- | --- |

| **Web Dashboard refuses to render layout components** | Verify that your hosting device and client terminal share the identical network domain. Double-check serial traces for IP updates. |

| **Sensor data tracks zero boundaries or locks at `999 cm` constant limits** | Confirm the VCC path leads to a **5V line**, as down-scaling to 3.3V power leaves the sonic transceivers unstable. |

| **Buzzer emits continuous alerts regardless of obstacle positions** | Verify that the module in place is a true **Active Buzzer** (which triggers via direct digital state lines) and check that the signal wire is not loose. |

| **RGB module shows the wrong color values** | Swap the target connection leads on GPIO 12 and 13; the pin configurations for the red and green legs may be reversed. |

---

## 11. Conclusion

This deployment showcases an elegant, low-latency industrial-style safety application powered by the ESP32 platform. By combining real-time hardware tracking with non-blocking software architectures and modern web data APIs, the system provides both reliable local control and highly detailed remote monitoring. It serves as a strong foundation for building multi-tier automation security grids, robotics systems, and advanced interactive smart industrial devices.