# DHT Sensor Interfacing via Blynk IoT

## Objective

Enable real-time monitoring of temperature and humidity using a DHT sensor and the Blynk IoT dashboard.

## Components Required

  - ESP32 development board

  - DHT11 Temperature & Humidity Sensor

  - Breadboard

  - Jumper wires

## Software Requirements

- Arduino IDE

- ESP32 board support installed

- Latest Blynk IoT library

- DHT sensor library by Adafruit

- Adafruit Unified Sensor library

## Circuit Connections

  - DHT Data pin → GPIO 4

  - DHT VCC → 3.3V pin

  - DHT GND → GND

## Functional Overview

ESP32 connects to Blynk Cloud; a non-blocking timer triggers sensor readings every 2 seconds; temperature and humidity values are sent to virtual pins for real-time dashboard visualization.

## Source Code

#define BLYNK_TEMPLATE_ID "TMPLxxxxxx"

#define BLYNK_TEMPLATE_NAME "DHT"

#define BLYNK_AUTH_TOKEN "BPOK3C1-FQCkzVlnF0oFhX0iZZuHCJvq"

// Print messages to Serial Monitor

#define BLYNK_PRINT Serial

#include <WiFi.h>

#include <WiFiClient.h>

#include <BlynkSimpleEsp32.h>

#include <DHT.h>

// Your WiFi credentials

char ssid[] = "NSUT_WIFI";

char pass[] = "";

// DHT Sensor Setup

#define DHTPIN 4          // Digital pin connected to the DHT sensor (GPIO 4 / D4)

#define DHTTYPE DHT11     // Type of DHT sensor (DHT11, DHT22, etc.)

DHT dht(DHTPIN, DHTTYPE); // Starting the sensor through library

// Create a timer object

BlynkTimer timer;

// Function to read sensor and send data to Blynk

void sendSensor()

{

float h = dht.readHumidity();

float t = dht.readTemperature(); // or dht.readTemperature(true) for Fahrenheit

// Check if any reads failed and exit early (to try again)

if (isnan(h) || isnan(t)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

// Send data to Blynk Virtual Pins

// Assuming V0 is set up for Temperature and V1 for Humidity in your Blynk Datastreams

Blynk.virtualWrite(V0, t);

Blynk.virtualWrite(V1, h);

// Print to Serial monitor for debugging

Serial.print("Temperature: ");

Serial.print(t);

Serial.print(" °C  |  Humidity: ");

Serial.print(h);

Serial.println(" %");

}

void setup()

{

// Debug console

Serial.begin(115200);

// Initialize Blynk

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// Initialize DHT sensor

dht.begin();

// Setup a function to be called every 2 seconds

// DHT11 requires at least 2 seconds between readings

timer.setInterval(2000L, sendSensor);

}

void loop()

{

Blynk.run();

timer.run(); // Initiates BlynkTimer

}

## How It Works

- BlynkTimer schedules the sensor reading at fixed intervals without pausing the ESP32 connection.

- The DHT library processes the digital 1-wire signal from the physical sensor into float variables.

- Blynk.virtualWrite handlers push the temperature and humidity data to the corresponding cloud datastreams.

- Changes reflect in real time on the configured widgets

## Output Example

- On Blynk dashboard:

- Two Gauge or Value Display widgets for Temperature and Humidity.

- Dashboard displays current climate data updating automatically.

## Applications

- Weather stations

- Smart home climate monitoring

- Greenhouse automation and environmental control

## Troubleshooting Tips

- Verify the sensor is powered by 3.3V, as sending 5V logic to ESP32 GPIOs can cause damage.

- If readings show "NaN" (Not a Number), double-check the wiring and ensure a pull-up resistor is used if needed.

- Ensure BLYNK_TEMPLATE_ID and BLYNK_TEMPLATE_NAME are placed at the very top of the sketch.

- Check Blynk auth token and WiFi network settings.