ESP32 Basic IR Signal Detector Using IR Receiver Module

Objective

To design and implement a basic IR signal detector using an ESP32 and an IR receiver module. The ESP32 reads the digital output of the IR receiver and detects an infrared signal whenever the signal level goes LOW.

Theory

An IR receiver module converts infrared light signals into a digital voltage. The ESP32 monitors this voltage using a digital input pin (GPIO 14). Most IR receiver modules use an active-low configuration, meaning the output pin drops to 0V (LOW) when an IR signal is detected and remains at 3.3V (HIGH) when no signal is present.

Components Required

- ESP32 Development Board

- IR Receiver Module

- Jumper Wires

- USB Cable

- Breadboard (optional)

Circuit Connections

VCC (+) -> 3.3V

GND (-) -> GND

OUT (S) -> GPIO 14

DO (D) -> Not Connected

Program

#include <WiFi.h> #include <WebServer.h> // === Wi-Fi Credentials === const char* ssid = "NSUT_WIFI"; const char* password = ""; // === IR Receiver Pin === const int irPin = 14; // Connect OUT from IR receiver to GPIO14 // === Web Server === WebServer server(80); unsigned long lastIRTime = 0; void handleRoot() { String html = "<!DOCTYPE html><html><head><meta http-equiv='refresh' content='1'><title>IR Receiver</title></head><body>"; html += "<h2>ESP32 Basic IR Detection</h2>"; if (millis() - lastIRTime < 500) { html += "<p style='color:green;'>IR Signal Detected!</p>"; } else { html += "<p style='color:red;'>No IR Signal Recently</p>"; } html += "</body></html>"; server.send(200, "text/html", html); } void setup() { Serial.begin(115200); // IR pin setup pinMode(irPin, INPUT); // Connect to Wi-Fi WiFi.begin(ssid, password); Serial.print("Connecting to WiFi"); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println("\nWiFi connected!"); Serial.print("ESP32 IP address: "); Serial.println(WiFi.localIP()); // Start Web Server server.on("/", handleRoot); server.begin(); Serial.println("Web server started."); Serial.println("IR detection ready on pin " + String(irPin)); } void loop() { server.handleClient(); // Check IR pin for LOW signal if (digitalRead(irPin) == LOW) { lastIRTime = millis(); Serial.println("IR light detected!"); delay(100); // Simple debounce } }

Working Principle

1. Monitor IR pin for incoming digital signal.

2. Read digital value from the IR receiver.

3. Display value on Serial Monitor.

4. Check if signal level goes LOW.

5. Print 'IR light detected!' when signal is received.

Procedure

1. Make the connections.

2. Upload the code.

3. Open Serial Monitor (115200 baud).

4. Observe readings.

5. Point an IR remote at the sensor and press a button.

6. Check web server output via local IP.

Expected Output

No Signal:

(Web page shows No IR Signal Recently)

IR Signal Detected:

IR light detected!

(Web page updates status)

Applications

Remote control systems, security systems, wireless data transfer, obstacle detection, proximity sensing.

Result

The ESP32 successfully detects infrared signals by reading the digital state changes of the IR receiver pin.

Conclusion

The project demonstrates how to capture external infrared pulses using an ESP32 microchip and handle real-time conditions via web-based servers.