# ESP32 Clap Detector Using Analog Sound Sensor

Objective

## Components Required

ESP32 Development Board

Analog Sound Sensor Module

Jumper wires

USB cable

Breadboard (optional)

## Circuit Connections

| Sound Sensor | ESP32 |
| --- | --- |
| VCC (+) | 3.3V |
| GND (-) | GND |
| AO (A) | GPIO 34 |
| DO (D) | Not Connected |

## Code

const int micPin = 34;
const int threshold = 0;

void setup() {
  Serial.begin(115200);
}

void loop() {
  int sound = analogRead(micPin);
  Serial.println(sound);

  if (sound > threshold) {
    Serial.println("Clap detected!");
    delay(300);
  }

  delay(10);
}

## Working Principle

1. The sound sensor converts sound into an analog voltage.
2. The ESP32 reads the analog value from GPIO 34.
3. The value (0–4095) is displayed on the Serial Monitor.
4. If the value exceeds the threshold, the message 'Clap detected!' is printed.
5. A 300 ms delay prevents multiple detections from a single clap.

## Testing Procedure

• Upload the code.
• Open the Serial Monitor at 115200 baud.
• Observe the values in a quiet room.
• Clap near the sensor and note the peak value.
• Adjust the threshold to a value above the background reading and below the clap peak.

## Conclusion

This project demonstrates clap detection using an ESP32 and an analog sound sensor. The analog output is monitored continuously, and a clap is detected whenever the sound level exceeds a user-defined threshold.