# ESP32 Clap Detector Using Analog Sound Sensor

## Objective

To design and implement a clap detector using an ESP32 and an analog sound sensor. The ESP32 reads the analog output of the microphone and detects a clap whenever the sound level exceeds a predefined threshold.

## Theory

A sound sensor converts sound into an analog voltage. The ESP32 reads this voltage using its 12-bit ADC (0-4095). When the measured value exceeds a preset threshold, a clap is detected.

## Components Required

- ESP32 Development Board
- Analog Sound Sensor Module
- Jumper Wires
- USB Cable
- Breadboard (optional)

## Circuit Connections

VCC (+) -> 3.3V
GND (-) -> GND
AO (A) -> GPIO 34
DO (D) -> Not Connected

## Program

const int micPin = 34;
const int threshold = 0;

void setup() {
  Serial.begin(115200);
}

void loop() {
  int sound = analogRead(micPin);
  Serial.println(sound);

  if (sound > threshold) {
    Serial.println("Clap detected!");
    delay(300);
  }

  delay(10);
}

## Working Principle

1. Capture sound.
2. Read analog value.
3. Display value on Serial Monitor.
4. Compare with threshold.
5. Print 'Clap detected!' when threshold is crossed.

## Procedure

1. Make the connections.
2. Upload the code.
3. Open Serial Monitor (115200 baud).
4. Observe readings.
5. Clap near the sensor.
6. Adjust threshold if required.

## Expected Output

Quiet:
1500
1498

Clap:
2870
Clap detected!

## Applications

Smart home automation, clap-operated switches, security systems, noise monitoring, IoT projects.

## Result

The ESP32 successfully detects a clap by comparing the analog sound level with a predefined threshold.

## Conclusion

The project demonstrates clap detection using the ESP32 and an analog sound sensor with adjustable software threshold.